ChE 382 – Chemical Engineering Laboratory II- Spring 2017

Student Learning Outcomes 1, 3 (written and oral), 4, 5, 6, 7

Rubric for assessment for Pre-lab Report, Final Lab Report, Team Skills Assessment, SACHE Safety Tutorial; Dust Explosion, Safety Quiz: Industrial Hazards

Average scores showed most students were very strong in experimental preparation, discussion of results, lab and industrial safety, sample calculations, data tabulation and error analysis.  A large majority were satisfactory in analyzing the experimental data and drawing conclusions.  Most students were also strong in teamwork/team skills.

Assessment Rubric for Lab reports (pre-lab and final lab report sections combined)

| Report Section | SLO’s | (1) Unsatisfactory | (2) Developing | (3) Satisfactory |
| --- | --- | --- | --- | --- |
| 1.Introduction | SLO 1, 3 | Main aspects of the lab experiments are not addressed; no experimental objective, experimental description or discussion of industrial applications, poor grammar and usage | Several main aspects of lab experiment not addressed or not stated in a clear and concise manner. | Main aspects of lab experiment addressed and clearly stated; minor grammar and usage issues. |
| 2. Experimental: Apparatus, Materials & Supplies, Procedure | SLO 3, 6 | Incomplete process diagram, missing materials/supplies list, incomplete or vague experimental procedure | Adequate process diagram, material/ supply list, experimental procedure; several problems or mistakes | Complete process diagram, material/ supply list, experimental procedure with minor problems |
| 3. Project Safety Evaluation | SLO 3, 4, 7 | Missing or lacking an understanding of major/ minor safety hazards and safe operating conditions | Safety understanding, missing some major safety hazards | Safety understanding and important safety hazards, missing some minor hazards |
| 4. Executive Summary | SLO 3 | Illegible or incoherent summary of conclusions, poor grammar and formatting | A few problems with summary of conclusions, legibility or neatness | Minor problems with summary of conclusions, grammar, formatting, neatness |
| 5. Results | SLO 1, 3, 6 | Drawings, graphs and tables unacceptable, confusing, or missing | Several deficiencies in drawings, graphs and tables. Figures and tables not referred to in text. | Only a few deficiencies with drawings, graphs and tables |
| 6. Discussion | SLO 1, 3, 6 | Main points and trends from results are missing or not discussed clearly; lack of understanding of the engineering reasoning behind the analysis | Several main points and trends are missing or not discussed clearly; some understanding of engineering reasoning behind analysis | Only a few points or trends are missing or not discussed clearly; a substantial understanding of the engineering reasoning behind analysis |
| 7. Conclusions | SLO 3, 6 | No significant conclusions drawn, Limitations of results and future work not discussed. | Cursory conclusions drawn from data and calculations. Some discussion of limitation of results and future work. | Valid conclusions drawn from data and calculations. Future work addressed in relation to limitations or uncertainties. |
| 8. References | SLO 3, 7 | Seeks no extra information other than what is provided by instructor | Seeks information from a few sources, mainly from the textbook or instructor | Seeks information on problems from multiple sources |
| 9. Data Tabulation | SLO 1, 3, 6 | Experimental data tables and graphs unacceptable or missing | Several deficiencies with data tables and graphs | Only a few deficiencies in data tables and graphs |
| 10. Sample Calculations | SLO 1, 3, 6 | Calculations are incorrect, presented in a confusing way and do not link with specific experimental results | Some calculations are incorrect or presented in a confusing way | One or two calculations are incorrect or presented in a confusing way |
| 11.Error Analysis | SLO 1, 3, 6 | Cursory and qualitative description of errors | Qualitative analysis of error with some calculations | Quantitative and qualitative error analysis, missing some minor calculations |

Assessment Grading for Lab reports (pre-lab and final lab report sections combined)

| Report Section | SLO’s | (1) Unsatisfactory | (2) Developing | (3) Satisfactory |
| --- | --- | --- | --- | --- |
| 1. Introduction | SLO 1, 3 | 0-2 | 3 | 4-5 |
| 2. Experimental: Apparatus, Materials & Supplies, Procedure | SLO 3, 6 | 0-11 | 12-15 | 16-20 |
| 3. Project Safety Evaluation | SLO 3, 4, 7 | 0-5 | 6-7 | 8-10 |
| 4. Executive Summary | SLO 3 | 0-5 | 6-7 | 8-10 |
| 5. Results | SLO 1, 3, 6 | 0-8 | 9-11 | 12-15 |
| 6. Discussion | SLO 1, 3, 6 | 0-8 | 9-11 | 12-15 |
| 7. Conclusions | SLO 3, 6 | 0-6 | 7-8 | 9-10 |
| 8. References | SLO 3, 7 | 0-2 | 3 | 4-5 |
| 9. Data Tabulation | SLO 1, 3, 6 | 0-8 | 9-11 | 12-15 |
| 10. Sample Calculations | SLO 1, 3, 6 | 0-8 | 9-11 | 12-15 |
| 11.Error Analysis | SLO 1, 3, 6 | 0-8 | 9-11 | 12-15 |

Assessment Rubric for Team Member Skills

| Performance Indicator (PI) | SLO’s | (1) Unsatisfactory | (2) Developing | (3) Satisfactory |
| --- | --- | --- | --- | --- |
| 12.Quality Work | SLO 5 | Has a poor grasp of chemical engineering principles and poor writing skills | Has some knowledge of chemical engineering principles and writing skills | Has good knowledge of chemical engineering principles and writing skills |
| 13. Teamwork | SLO 5 | Hides in the background; only participates if strongly encouraged Routinely fails to prepare any plans, does a minimal amount of task work | Occasionally plans and carries out lab tasks. Occasionally takes charge when not in the position to lead | Demonstrates the ability to assume a designated role in the group, plan tasks, carry out tasks and contributes to success of team |
| 14. On-time tasks | SLO 5 | Consistently hands in task late to the team | Occasionally is late in handing in tasks | Hands in tasks (lab report sections) on time to team |

Team Member Assessment Evaluation Form:

Assessment Rubric for Safety Assignments

| Performance Indicator (PI) | SLO’s | (1) Unsatisfactory | (2) Developing | (3) Satisfactory |
| --- | --- | --- | --- | --- |
| 15. Understanding industrial safety and practices | SLO 4 | Missing or lacking an understanding of industrial safety practices | Has some understanding of industrial hazards and safety practices | Has a good understanding of industrial hazards and safety practices Example: Passed a safety tutorial (SACHE) |
| 16. Understanding of lab safety and safety evaluations | SLO 4 | Missing or lacking an understanding of major/ minor safety hazards and safe operating conditions | Missing some major/minor aspects of laboratory safety practices | Satisfactory understanding of laboratory safety covered. Example: Understands a laboratory Chemical Hygiene Plan |

Safety Assignment: SACHE Tutorial “Dust Explosion”

Safety Assignment:  Industrial Hazards

Laboratory safety is an essential part of the Chemical Engineering Laboratory.

Safety Assignment I :

1) Read the selected chapters from Guidelines on Hazardous Chemical Safety from JT Baker Safety Course. (see pdf file)

Chap 3:Toxic chemicals

Chap 8: Flammable chemicals

Chap 9: Corrosive chemicals

Chap 11: Reactive chemicals

Chap 12: Insidious hazards

Chap 13: Compressed gases

2) Complete the safety quiz.

DUE: January 19th, beginning of lab period.

SAFETY QUIZ:CHE382-Chemical Engineering Laboratory II

Please answer the following questions testing your knowledge from 6 selected chapters of the JT Baker Hazardous Chemical Safety document; Toxic Chemicals, Flammables, Corrosive Chemicals, Reactive Chemicals, Insidious Hazards and Compressed Gases.  Be brief but concise in your answers.  Open book/open notes and team discussion is allowed but answers must be in your own words.  (5 points per question)

- What is the general definition of a toxic chemical?

Any chemical which, when ingested, inhaled, absorbed, injected into or developed within the body, in relatively small amounts, by its chemical action, may cause damage to structure or disturbance to function.

- Name the general types of damage that a toxic chemical can cause:

Local toxicity, systemic toxicity, acute toxicity, chronic toxicity

- Name two parameters that can effect toxicity:

Two from list on page 3-3: Mode of entry, physical condition, dose, duration, sensitivity, combined effects, stress, others (must list)

- What is the DOT (Department of Transportation) definition of a nonflammable compressed gas?

A nonflammable material having a pressure greater than 40 psia at 70°F or 104 psia at 130°F.

- a) How does one determine how much material is remaining in a non-liquified gas cylinder?

Read the cylinder pressure gauge reading which is proportional to the amount of gas remaining inside the cylinder

b) On page 13-10, which pressure gauge on the two-stage regulator will give you the cylinder pressure reading?

The pressure gauge on the left with pressure readings from 500-3000 psi

- Other than a single compressed gas, what else can be stored inside a compressed gas cylinder?

Liquefied gas, a gas dissolved in a liquid or solid, or a compressed gas mixture

- Why do insidious hazards represent a problem to workers?

They represent a problem because the workers may not be aware that a problem exists until chronic, systemic poisoning has occurred.

- What are the four general categories of insidious hazards?

Unanticipated chemical reactions, unanticipated toxicity, improper facility design, improper handling of equipment and chemicals.

- Name 2 control measures that can be implemented to reduce insidious hazards in the laboratory.

See list of 8 control measures on page 12-13

- What is the definition of a reactive chemical?

Reactive chemicals are substances which enter into violent reactions during which the spontaneous liberation of heat and/or gases is too rapid to be safely dissipated by the surroundings.

- Name the two groups and their sub-headings used to categorize reactive chemicals:

Unstable compounds (Explosives, others)

Unstable mixtures (Explosives, water-reactive compounds, air-sensitive compounds, other mixtures of oxidizing agents with reducing agents)

- List two precautionary measures for handling and/or storing reactive chemicals.

See list on page11-8 or short paragraph on 11-7.

- How are corrosive chemicals generally defined?

Corrosive chemicals are substances that, by direct chemical action, are injurious to body tissue or corrosive to metal.

- How do corrosive chemicals act on body tissues?

They act through direct contact with skin or eyes, inhalation or ingestion, and by direct chemical reaction, dissolution of essential components, destruction of protein or disruption of cell membranes.

- True or False:  Concentrated alkaline solutions have a more corrosive effect on tissue than most strong inorganic acids.

True

(Strong alkalies gelatinize tissue, forming soluble compounds and thereby produce deep painful, penetrating burns)

- Name the six categories of common corrosive liquids.

Inorganic acids, organic acids, other inorganics, caustic solutions, organic solvents, other organics.

- What category of corrosive solids presents the greatest hazard because of their frequent use in industry?

Caustic alkalies

- Name the three components that make up the FIRE TRIANGLE:

Fuel, oxidizer, ignition source (or energy)

- What category of flammables is the most common source of fires in industry?

Flammable solvents

- List two common safety problems and two safety control measures for flammable solvents.

See lists on page 8-11

BONUS QUESTION:

Where is the eyeglass cleaning station located in the Unit Operations Laboratory (Room 120)?